Members:
Kien Olivar
Courtney Hagen
Adriano Valenga Carneiro
Troy Krahn
Sergio Garcia Medina

Capstone Project

Team: SANstorm

SAN Testing Environment
(Storage Area Network)

Contents

Overview	2

Project Members and Roles	2

Scope of Project	2

SAN Inventory	3

Unit 17	3

Unit 15	3

Unit 13	3

Unit 11	3

Unit 9	3

Unit 7	3

Unit 5	3

Rack Layout (Diagram)	4

Install Plan / Activity Plan	5

Testing Plan	7

Recommendations	9

SAN Security Concerns and Recommendations	9

Introduction	9

Network Security	9

Hardware Security	9

Backup & Recovery	9

Student Access	9

Hardware Recommendations	10

SAN Controller (Dell Powervault MD3820i)	10

Disk Enclosures (Powervault MD1400)	11

Disk Drives	11

RAID Levels	11

Network Recommendations	12

Other Recommendations	14

References	15

# Overview

The purpose of this document is to show our plan as to how we started our initial install, configuration, and testing of the SAN project. This includes an inventory of the equipment provided to us and creating a rack diagram showing the layout of the SAN. This document also includes a recommendation plan based on issues and concerns we encountered while working on the project such as security concerns, hardware, network, and other recommendations.

# Project Members and Roles

This section contains the information of the project members and their roles on the project.

- Kien Olivar			- Project Lead/Diagrams/Documentation

- Courtney Hagen		- Security Lead/Researcher/Documentation

- Adriano Valenga Carneiro	- Tech Team/Researcher

- Troy Krahn			- Tech Team/Researcher

- Sergio Garcia Medina		- Tech Team/Researcher

# Scope of Project

The objective of this project is to create a SAN testing environment that could be used for possible future implementation of a SAN in the Student Data Center which the students could use to store and share files. With SAN implemented, students will get a fast, reliable, manageable, and high-availability transfer of data making it easier for students to work with their project at the campus.

# SAN Inventory

This section contains the hardware specifications of our SAN.

## Unit 17

| Model Name | HP StorageWorks AIO1200 |
| --- | --- |
| Drives | 12 x 300GB 3.5” HP SAS Drives (3600GB) |
| Processor/Cores | Dual-Core Intel Xeon 2.67 GHz/1066 FSB with 4MB (2 x 2MB) L2 Cache |
| Memory | 2GB of PC2-5300 (667MHz) Unbuffered ECC DDR2 SDRAM (2 x 1GB) |

## Unit 15

| Model Name | HP StorageWorks 2000 Modular Smart Array (MSA2000) |
| --- | --- |
| Drives | 12 x 450GB 3.5” HP SAS Drives (5400GB) |

## Unit 13

| Model Name | HP StorageWorks 2000 Modular Smart Array (MSA2000) |
| --- | --- |
| Drives | 12 x 450GB 3.5” HP SAS Drives (5400GB) |

## Unit 11

| Model Name | HP StorageWorks 70 Modular Smart Array Enclosure (MSA70) |
| --- | --- |
| Drives | 25 x 146GB 2.5” HP SAS Drives (3650GB) |

## Unit 9

| Model Name | HP StorageWorks 70 Modular Smart Array Enclosure (MSA70) |
| --- | --- |
| Drives | 25 x 146GB 2.5” HP SAS Drives (3650GB) |

## Unit 7

| Model Name | HP StorageWorks 70 Modular Smart Array Enclosure (MSA70) |
| --- | --- |
| Drives | 16 x 146GB 2.5” HP SAS Drives (2336GB) |

## Unit 5

| Model Name | HP StorageWorks 60 Modular Smart Array Enclosure (MSA60) |
| --- | --- |
| Drives | 8 x 1TB 3.5” HP SAS Drives (8TB) |

# Rack Layout (Diagram)

This section shows the layout of the SAN on the Rack.

# Install Plan / Activity Plan

This section contains our Server/SAN Install Plan.

| Activity List/Plan |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Project: Capstone – SAN Testing Environment |  |  |  |  | Date: March 12, 2020 |
| Activity ID No. | Activity Name | Description of Work |  |  | Responsibility |
| A001 | Documentation required for install and configuration. | - Install Plan Document (Install Plan.docx) |  |  | Courtney Hagen |
| A002 | Resources required for install. | - Windows Server 2012 ISO |  |  | Adriano Valenga Carneiro |
| A003 | Documentation of settings and configurations required for implementation | Specify object names in preparation for implementation |  |  | Adriano Valenga Carneiro |
|  |  | Operating System: | Windows Server 2012 |  |  |
|  |  | System Hardware Configuration: | Primary HD | n/a |  |
|  |  |  | Additional HDs | n/a |  |
|  |  |  | RAM | n/a |  |
|  |  |  | Processors/Cores | n/a |  |
|  |  | Network | IP Address (Static/DHCP) | n/a |  |
|  |  |  | Subnet Mask | n/a |  |
|  |  |  | G/W Address | n/a |  |
|  |  |  | DNS Server Addresses (2) | n/a |  |
|  |  | Installation Mode | New Install |  |  |
|  |  | Time Zone: | Atlantic Time (UTC/GMT -04:00) |  |  |
|  |  | Server Type: | Server 2012 |  |  |
|  |  | Server “Administrator” Password | Tgkb2rVdGmT#G573 |  |  |
|  |  | Server Hostname | SAN01D316 |  |  |
|  |  | Domain Name | n/a |  |  |
|  |  | Backup Admin User: | SANAdmin |  |  |
|  |  | Password: | ZTBhm3?$rXYvcCWw |  |  |
|  |  | Additional User: | n/a |  |  |
|  |  | Password: | n/a |  |  |
|  |  | DSRM Password: | n/a |  |  |
| A004 | Additional configurations or settings applied | - Apply all critical Windows Updates to date. |  |  | Adriano Valenga Carneiro |
| A005 | Additional Software required as part of the initial install | - N/A |  |  |  |

# Testing Plan

This section contains our testing plan for our SAN to confirm that our SAN is working as intended and has no errors.

| Project (Activity) for Testing: Initial Install and Configuration Testing |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date: February 18, 2020 |  |  |  |  |  |  |  |
| Test ID | Description | Action | Expected Result | Responsibility | Status | Repeat Test | Notes: |
| T001 | System boots backup with no errors | Reboot the server and let it go through its regular boot sequence | System displays no errors while/after booting | Tech Team | Passed |  |  |
| T002 | OS shows login screen and takes user input | Enter user login credentials | User can successfully login | Tech Team | Passed |  |  |
| T003 | Correct version of Windows Server is installed | Go to Control Panel System and Security System | System version matches the one in the install plan (Windows Server 2012) | Tech Team | Passed |  |  |
| T004 | Windows is activated | Open Server Manager Local Server and check Product ID | Product ID shows that Windows is activated | Tech Team | Passed |  |  |
| T005 | Check Network Connectivity | Open a web browser and try to access NSCC.ca | NSCC.ca website can be accessed | Tech Team | Passed |  |  |
| T006 | Windows Updates | Go to Windows Update and select Check for updates | Windows Update screen shows there are no more update available | Tech Team | Passed |  |  |
| T007 | Time Zone | Compare system date and time to phone or a watch. Make sure Time Zone is set to Atlantic Time (Canada) | System date and time matches phone date and time. Time zone set to Atlantic Time (Canada) | Tech Team | Passed |  |  |
| T008 | Remote Desktop | Remote-in to the SAN from a different computer on the network. | Can successfully remote in. | Tech Team | Passed |  |  |

# Recommendations

This section contains all the SAN recommendations our group came up with based on the equipment that was provided to us and the concerns and issues we have encountered while working on the project.

## SAN Security Concerns and Recommendations

### Introduction

Ultimately, our group’s implementation of a SAN using the hardware we were provided posed great difficulty in securing. This is because of the age and functionality of the hardware. There were also aspects that would have been implemented had we not lost physical access to the SAN towards the end of the project. These suggestions are based on what we would have liked to have done but were not necessarily able to.

### Network Security

Due to the requirement of network connection between the SAN and the user’s environment and the need to be able to manage the SAN over the network for the administrator, the SAN has unique network security requirements.

Management access should be provided over a separate management VLAN as to not allow end users potential access to modify anything. This will also require configurations between switches used in the SAN to provide secure connections. Also, in terms of management securely connecting, there should be a verification of password encryption for the management appliance. Going above that, the management appliance’s default port should be changed if possible.

For the use of the SAN in Net 172, we recommend the implementation of Active Directory authentication. This will provide security for the contents and access to the SAN. Only certain users or computers could be granted access to the SAN contents if desired.

### Hardware Security

One of the major issues we encountered was the lack of updates available for the hardware we had to use for the SAN test environment. A recommendation we have going forward is to ensure regular BIOS and firmware updates.

A feature for hardware security that we did not implement but that we highly recommend is the encryption of the hard drives. This will prevent any unauthorized access if a hard drive were to be taken.

### Backup & Recovery

We recommend that half of the available drives be backup for the other half of the drives. Due to the nature of the intended purpose, offsite backups may not be necessary.

### Student Access

Permissions within the folders stored on the SAN will provide security and prevent unauthorized access to certain files and folders and should be utilized as the primary method to grant students access to the SAN.

## Hardware Recommendations

During this project, we faced some issues with the hardware we were given. Some of the issues we faced included

- The hardware was in excess of 10 years old

- The SAN controller would not run versions of Windows Server past 2012R2

- Management tools for the SAN controller were only supported on Windows Server 2003, any required frameworks were also out of date

- Drivers and firmware updates for the hardware required access which we did not have

- Network interfaces were only gigabit, so the speed of the SAN would not be ideal for a production environment

Given the issues we faced, we have come up with some recommendations for current hardware that would serve well for a SAN environment.

### SAN Controller (Dell Powervault MD3820i)

For the main SAN device, we recommend the Dell Powervault MD3820i. In a production environment, redundancy is an important factor to consider. Most SAN devices will have dual controllers (or have the option of installing a second), so if one were to fail the other can pick up the workload with minimal or no downtime. The Powervault MD3820i can have either single or dual controllers. Each controller has 2 12Gb Mini SAS connectors for connecting disk enclosures and has a dedicated management interface. The controller modules each have 2 dedicated 10Gb Ethernet interfaces for iSCSI, which is considerably better than the hardware we were using.

Figure 1 Diagram of controller connectors, showing 10Gb Ethernet interfaces (1), Mini SAS connectors (14) and out of band management port (8). Image Credit: https://topics-cdn.dell.com/pdf/powervault-md3800i_owners-manual4_en-us.pdf

The MD3820i chassis can hold up to 24 2.5” drives or 12 3.5” drives depending on the model chosen, with the option of SAS or SSD drives. Solid state drives can be used in the 3.5” model with the use of adapters.

### Disk Enclosures (Powervault MD1400)

The additional disk enclosures are simply JBODs (Just a Bunch of Disks) that will connect to the Powervault MD3820i via Mini SAS connectors. The MD3820i will handle the creation of RAID arrays and virtual disks, as well as creation of LUNs and iSCSI targets.

### Disk Drives

The type of disk drives to be used would depend upon the ultimate use case of the SAN. For bulk storage of files, standard SAS or SATA hard drives would be enough. However, if the SAN were to be used for running and storing students’ virtual machines, solid state drives would be a better choice. SSDs have much faster access times with virtually no delay, since there is no head seeking over a spinning platter as in regular hard drives. Virtual machines would run and be much “snappier” from the student’s point of view.

### RAID Levels

We would recommend the use of RAID 10 for the SAN since this RAID level offers both mirroring and striping. The only downside to RAID 10 is some storage space is lost to accommodate for the mirroring of the data. There is a significant increase in read and write speeds. Assuming the SAN controller is fully populated with 1TB disks, we would have the following total amounts of storage (excluding any attached DAS devices):

- 5.5TB usable with 12 3.5” 1TB drives

- 10.9TB usable with 24 2.5” 1TB drives

## Network Recommendations

For our test environment, the networking equipment provided to us would not affect the speeds in which data transfer would occur in the test network, in theory. With the removal of the Dell PowerEdge R900 server as the main controller for our SAN, there was no longer a need for a network connection between the main unit and the storage arrays. The HP StorageWorks AiO1200 unit became the host controller, and all other 6 storage array devices (2x MSA2000 and 4x MSA70) connect to it via Serial Attached SCSI connectors (SAS – 2), which allows, in theory, for up to 6gbps speeds transfer in between devices, through the use of SFF-8088 External Mini SAS cables.

As the HP StorageWorks AiO1200 unit allows for our SAN to be completely self-contained – Operating System, array controller and the test workstations (Virtual Machines running on the host for testing transfers in the environment) in a single device, the speed in which data can be moved to and from the host would rely solely on the speed of the SAS connections. In our test scenario, the fastest transfer speeds would be up to 6gbps.

Aside from a test environment, this is not a practical scenario. In a real-world application, a company/enterprise would have multiple devices accessing the SAN, for several different reasons, and from a number of different locations. And for that to happen, those devices would require network access to the SAN.

The limiting factor in data transfer speeds for this usage is how fast data can be moved through the network. At an earlier stage of our test environment, the main controller that was to be used, the Dell PowerEdge R900 server, would be connected to the SAN via a wired network connection, by doing link aggregation (NIC teaming in the case of a Windows Server OS) on all 4 NICs, providing up to 4gbps of bandwidth – in theory. The HP StorageWorks AiO1200 unit could also have its NICs teamed to allow for the same 4gbps of bandwidth. The connection between the two units alone – Dell R900 and HP AiO1200, would be a slight bottleneck to the environment, as the theoretical speeds in which data can be transferred in the network are 2gbps slower than the speed of the SAS connectors.

Even with the loss of 2gbps worth of possible/theoretical throughput due to the limitations of the technology we have access to, most companies which would have a use for such SAN in their environments would not suffer from low transfer speeds. For most scenarios, the use of network cards with gigabit connections, and switches that allow for link aggregation, would provide satisfactory performance.

For a smaller niche, and very specific applications that require ultra-high speed bandwidth, such as large datacenters, large-scale VM servers which can perform read/write disk operations straight from a network location; or companies that need video editing capabilities of raw video files directly from their storage location, other technologies have to be used in order for the network connections not to become a degrading performance issue. The use of fiber channel cables, fiber channel switches and enterprise grade solid state drive (SSD) arrays make up for the fastest available option in the market, nowadays. This allows for a what is called a Switched Fabric design, the topology for a storage are network concept which is like modern Ethernet implementations, but with much higher speeds and better fault tolerance. This costly, but highly efficient design, allows for speeds of up to 25.6 gbps per direction or 51.2 gbps full duplex, with the use of the 256GFC “Gen7” protocol.

As demonstrated, there are multiple ways in which this technology can be used. Determining what kind of equipment will be used must be closely linked to the business needs. Datacenter like speeds, reliability and fault tolerance aspects are directly connected to the cost of the technology.

In the event that such SAN environment is to be implemented on the Student Datacenter, located on room D316 of the IT Campus, the storage space created could provide students from the Systems Management and Security program with enough disk space to store the virtual machines used throughout the course. But in order for the network to accommodate the traffic generated by the amount of data necessary to read or transfer VM files from the arrays to the workstations in D316, all of the switches in the Student Datacenter would have to be upgraded to at least gigabit switches. With the current setup, the 100mbps ports on both Cisco and Dell switches would be the bottleneck in the entire system. The design and general idea behind the solution is feasible, and would, theoretically, still work. But the limitations of the current networking hardware would make for a scenario in which the main functionality would not be viable.

## Other Recommendations

It was during our project development that our Team realized how Power Consumption turned to be a big challenge to confront and solve.

When all information was gathered, plans made, and tasks assigned to begin building the SAN solution using the designated resources (devices and software) our team faced a critical point where we crossed with energy consumption issues. At a first approach all the storage units were tested individually and PASSED the marks (Server and Storage units have double power supply units for redundancy), it was then later when our team began building the solution and started configuring the R900 server and two Storage units that realized about how much wattage the system was consuming. Our team always worked closely with our customer (Student Datacenter Administrator – Reneé Beaudin) and found that the electric circuit used to connect, and power the SAN environment was close to maximum capacity.

The scenario described above prompted us to take a different approach to meet project outcomes and our team decided not to power up all of the units at the same time and rather do it using  the R900 Server, Controller unit and two Storage units.

As a conclusion, in terms of Power, as discussed with the Student Datacenter Administrator it is HIGHLY RECOMMENDED to find an alternate electrical circuit or power grid source with enough capacity for the entire SAN system.

Desired scenario.

Actual scenario.

Previous recommendation sections already informed about security, hardware and network requirements that should be considered for further references if a the project was to move forward by another team or group, It is important then to underline that redundancy is not only attached to SAN in terms of energy but also in terms of data traffic flow.

Our strong recommendation is to consider a second switch device to be integrated into the system, this second switch will ensure a reliable alternate channel to avoid network traffic interruption and provide access to information (a second path).

# References

Security Recommendations:

The Basics of SAN Security, Part I. (n.d.). Retrieved April 20, 2020, from 	https://www.enterprisestorageforum.com/sans/features/article.php/1431341/The-Basics-of-	SAN-Security-Part-I.htm

The Windows Server Hardening Checklist. (2019, December 5). Retrieved April 20, 2020, from 	https://www.upguard.com/blog/the-windows-server-hardening-checklist

Hardware Recommendations:

PowerVault MD3820i. (n.d.). Retrieved from https://www.dell.com/ai/business/p/powervault-	md3820i/pd

PowerVault MD1400 and MD1420 Direct-Attached Storage. (n.d.). Retrieved from 	https://www.dell.com/en-us/work/shop/povw/storage-md1420

Das, A. (2018, March 15). RAID levels 0, 1, 4, 5, 6, 10 explained. Retrieved from 	https://www.booleanworld.com/raid-levels-explained/

Network Recommendations:

Dell. (2019, April 10). Introduction to Fibre Channel Storage Area Networks (SAN). Retrieved from Dell.com: https://www.dell.com/support/article/en-ca/sln69071/introduction-to-fibre-channel-storage-area-networks-san?lang=en

Industry’s Fastest Storage Networking Speed Announced by Fibre Channel Industry Association ─ 64GFC and Gen 7 Fibre Channel. (2018, November 12). Retrieved from Intrado - Globenewswire: https://www.globenewswire.com/news-release/2018/11/12/1649822/0/en/Industry-s-Fastest-Storage-Networking-Speed-Announced-by-Fibre-Channel-Industry-Association-64GFC-and-Gen-7-Fibre-Channel.html

Robb, D. (2018, March 29). Storage Area Networks in the Enterprise. Retrieved from Enterprise Storage forum: https://www.enterprisestorageforum.com/storage-networking/storage-area-networks-in-the-enterprise.html

Wikipedia contributors. (2020, April 12). Fibre Channel. Retrieved from Wikipedia, The Free Encyclopedia.: https://en.wikipedia.org/w/index.php?title=Fibre_Channel&oldid=950452632

Wikipedia contributors. (2020, March 24). Serial Attached SCSI. Retrieved from Wikipedia, The Free Encyclopedia.: https://en.wikipedia.org/w/index.php?title=Serial_Attached_SCSI&oldid=947079799

Wikipedia contributors. (2020, February 28). Storage area network. Retrieved from Wikipedia, The Free Encyclopedia.: https://en.wikipedia.org/w/index.php?title=Storage_area_network&oldid=943041989